Datasheet

No.: 04FBAB2058K-DC 24-FB13B6+FB14B7-20°

Version: 2025-11-05-02

| ITEM: 04FBAB2058K | INPUT VOLTAGE(V) DC 24 | LENS FB13B6+FB14B7 | LENS DEGREE(°)θ1/2 20° |  | CCT 3000K |
| --- | --- | --- | --- | --- | --- |
|  |  | LIGHT SOURCE |  | 20×3W |  |
|  |  | CONSUMPTION(W) |  | 73.2 |  |
|  |  | OPERATING CURRENT(MA) |  | 3050 |  |
|  |  | POWER FACTOR(PF) |  | 1 |  |
|  |  | LUMINANCE(LM) |  | 5561 |  |
|  |  | COLOR RENDERING INDEX |  | RA: 80 |  |
|  |  | IK RATING |  | 10 |  |
|  |  | IP RATING |  | 68 |  |
| PRODUCT SPECIFICATIONS |  |  |  |  |  |
| FRONT COVER& HOUSING | Hard chromeplated molding shaped stainless steel 316L# |  |  |  |  |
| LIGHT WINDOW | step tempered glass. T=10mm |  |  |  |  |
| CABLE GLAND | IP68 PG-11 copper with nickel-coated |  |  |  |  |
| GASKET | Silicone Gasket |  |  |  |  |
| SLEEVE | PC/ABS ALLOYS |  |  |  |  |
| OPERATING RESTRICT | Temperature of water between -20˚C~40˚C Less than 2 meter depth |  |  |  |  |
| PCB | Excellent heat conductivity aluminum.coefficient of heatConductivity≥2.0w/mk |  |  |  |  |
| LED TYPE | SMD |  |  |  |  |
| LENS | OPTICS Lens,efficiency≥85% |  |  |  |  |
| GLASS PRESSURE CAPACITY | 1,934kgs |  |  |  |  |
| LED DRIVER | Built in on/off driver |  |  |  |  |
| BUILT-IN OVERHEAT SENSOR SYSTEM | Automatic Shutdown at 75°C |  |  |  |  |
| POWER CABLE OPTIONAL | H07RN-F 2×1.0mm2 with water-stopper, L=1m, 3m, 5m |  |  |  |  |
| DIMMING SUPPORT | External DALl supported TRIAC,0-10V, PWM via external driver on request |  |  |  |  |

WATER PROOF SOLUTION

PHOTOMETRIC

CERTIFICATES