Datasheet

No.: 05AGB0126-DC 12 24-B106-24°

Version: 2025-11-05-03

| ITEM: 05AGB0126 | INPUT VOLTAGE(V) DC 12 24 | LENS B106 | LENS DEGREE(°)θ1/2 24° |  | CCT 3000K |
| --- | --- | --- | --- | --- | --- |
|  |  | LIGHT SOURCE |  | 1×6W |  |
|  |  | CONSUMPTION(W) |  | 7.4 |  |
|  |  | OPERATING CURRENT(MA) |  | 12=616 24=305 |  |
|  |  | POWER FACTOR(PF) |  | 1 |  |
|  |  | LUMINANCE(LM) |  | 520 |  |
|  |  | COLOR RENDERING INDEX |  | RA: 80 |  |
|  |  | IK RATING |  | 07 |  |
|  |  | IP RATING |  | 68 |  |
| PRODUCT SPECIFICATIONS |  |  |  |  |  |
| FRONT COVER | Hard chromeplated molding shaped stainless steel 316# |  |  |  |  |
| HOUSING | Hard chromeplated molding shaped stainless steel 316L# |  |  |  |  |
| LIGHT WINDOW | PMMA 8mm |  |  |  |  |
| GASKET | Silicone Gasket |  |  |  |  |
| CABLE GLAND | IP68 PG-11 copper with nickel-coated |  |  |  |  |
| OPERATING RESTRICT | Temperature of water between -20˚ C~40˚ C Less than 2 meter deptht |  |  |  |  |
| PCB | Excellent heat conductivity aluminum.coefficient of heat Conductivity≥2.0w/mk |  |  |  |  |
| LENS | Optical Lens,efficiency≥85% |  |  |  |  |
| LED TYPE | COB |  |  |  |  |
| LED DRIVER | Built in on/off driver |  |  |  |  |
| POWER CABLE | Outside of luminaire (Without waterstopper) H07RN-F 2X1.0mm² L=3.0m |  |  |  |  |
| DIMMING SUPPORT | External DALl supported TRIAC,0-10V, PWM via external driver on request |  |  |  |  |
| WATER PROOF SOLUTION |  |  |  |  |  |

TILTABLE DIRECTIONS

PHOTOMETRIC

CERTIFICATES