Datasheet

No.: 08VHB0126-DC 24-B71-60°

Version: 2025-11-06-02

| ITEM: 08VHB0126 | INPUT VOLTAGE(V) DC 24 |  | LENS B71 | LENS DEGREE(°)θ1/2 60° |  | CCT 3000K |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | LIGHT SOURCE |  | 1×6W |  |
|  |  |  | CONSUMPTION(W) |  | 7.9 |  |
|  |  |  | OPERATING CURRENT(MA) |  | 330 |  |
|  |  |  | POWER FACTOR(PF) |  | 1 |  |
|  |  |  | LUMINANCE(LM) |  | 699 |  |
|  |  |  | COLOR RENDERING INDEX |  | RA: 80 |  |
|  |  |  | IK RATING |  | 08 |  |
|  |  |  | IP RATING |  | 65 |  |
| PRODUCT SPECIFICATIONS |  |  |  |  |  |  |
| HOUSING | 12# Die-casting aluminum powder coated |  |  |  |  |  |
| PAINTING | Powder coating, T= 60~80 μm. Adhesion of ISO class 1/ASTM class 4B |  |  |  |  |  |
| LENS | OPTICS reflector. efficiency≥85% |  |  |  |  |  |
| LED TYPE | COB |  |  |  |  |  |
| POWER CABLE | Outside of luminaire (Without waterstopper) H05RN-F 2×1.0mm², L=0.3m(12V/24V) |  |  |  |  |  |
| OPERATING RESTRICT | -20˚ C~40˚ C |  |  |  |  |  |
| ACCESSORIES(Optional) |  | SAFETY HANGER |  |  |  |  |
|  |  | Instructions for the use of safety hanger When wiring, install the safety hanger on the ceiling mount. There is no need to remove the hanger after completion, which can be used as a protection when removing the fitting. |  |  |  |  |

| PAINTING COLOR |  |
| --- | --- |
| Primary | Secondary |

PHOTOMETRIC

CERTIFICATES